BlueSim Training Guide

Air Compressor & Starting Air System

Engine Room Equipment

Two-stage reciprocating air compressor and starting air system simulator per SOLAS II-1/Reg.26 — dual compressors, air bottles, cross-connection, and ME/AE start testing.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Two-stage reciprocating air compressor and starting air system simulator per SOLAS II-1/Reg.26 — dual compressors, air bottles, cross-connection, and ME/AE start testing.

## Key Features

Dual compressor (No.1 and No.2) start/stop with inter-stage pressure and discharge temp gauges.

Air bottle 1 and 2 pressure gauges (0–30 bar) with live charging simulation.

Cross-connection valve for emergency bottle balancing.

ME and AE start test buttons with air consumption simulation and SOLAS start count.

Alarms: low bottle pressure (<20 bar warning, <15 bar alarm), high discharge temp, LO low pressure, safety valve lift.

Three scenarios: Charging from Empty, Compressor 1 Failure, Cross-Connection Procedure.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| STARTING AIR SYSTEM SCHEMATIC | Displays real-time system data, controls, and status indicators. |
| GAUGES & ALARMS | Displays real-time system data, controls, and status indicators. |
| VALVE LINEUP | Displays real-time system data, controls, and status indicators. |
| BOTTLE PRESSURE TREND | Displays real-time system data, controls, and status indicators. |
| SYSTEM STATUS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Air Compressor & Starting Air System — Technical Reference

### Regulatory Framework

#### SOLAS II-1/Regulation 26 — Starting Arrangements for Main Propulsion Machinery

Every ship shall be provided with efficient means for starting main propulsion machinery from a dead-ship condition.

**Starting air systems** for diesel engines must be capable of providing at least **12 consecutive starts** for reversible engines or **6 consecutive starts** for non-reversible engines.

**Two independent air receivers** are required, each of sufficient capacity.

Air receivers must be maintained at maximum pressure prior to starting manoeuvres in or near port.

Total starting air capacity: sufficient for at least the number of starts required by SOLAS.

#### SOLAS II-1/Regulation 26(4) — Protection Against Over-Pressure

Starting air system main lines shall be fitted with a safety valve set to maximum allowable working pressure.

Non-return valves (check valves) shall be fitted in starting air supply lines.

Provision for drainage of starting air mains and receivers.

#### MSC/Circ.647 — Guidance on Maintenance of the Starting Air System

Regular checks of starting air bottle drain traps.

Oil carry-over from compressor to air bottles is a fire hazard — oil separators are mandatory.

SOLAS requires regular testing of safety valves on air receivers.

Unattended machinery spaces (UMS) require auto-start logic for standby compressor.

#### SOLAS II-1/Regulation 30 — Dead-Ship Condition Starting

The emergency source of power must be able to start the main propulsion auxiliary systems.

For diesel ships, starting air represents the primary means of main engine starting.

#### Class Requirements

**DNV GL, LR, ABS**: Two starting air compressors required, at least one shall be independently driven (not from the main engine).

Compressor safety valve: set at maximum working pressure.

Air receiver safety valve: set at MAWP of receiver.

Minimum bottle pressure for reliable ME start: 20–22 bar (varies by engine design).

Bottles charged to full working pressure (30 bar typical) when in port or at sea.

---

### Starting Air System Components

#### Air Receivers (Bottles)

| Parameter | Typical Value |

|-----------|--------------|

| Working Pressure | 30 bar |

| Safety Valve Setting | 32 bar |

| Hydrostatic Test Pressure | 45 bar |

| Volume | 0.5–2.0 m³ each |

| Material | Carbon steel, class approved |

| Inspection Interval | 2.5 years (internal), 5 years (external) |

#### Compressor (Two-Stage Reciprocating)

| Parameter | Value |

|-----------|-------|

| Type | 2-stage reciprocating |

| Drive | Electric motor |

| Output | 100–150 bar·l/min (10–15 m³/h) |

| Max Working Pressure | 30 bar |

| Inter-stage Pressure | 6–8 bar |

| Cooling Method | Water-cooled (LT cooling water) |

| Lubricant | SAE 30 mineral oil (non-synthetic for fire risk) |

| Safety Valve (inter-stage) | Set at 9 bar |

| Safety Valve (final stage) | Set at 32 bar |

#### Starting Air Distribution

| Consumer | Typical Pressure Required |

|----------|--------------------------|

| Main Engine (ME) | 25–30 bar (direct start) |

| Auxiliary Engine (AE) | 20–25 bar |

| Diesel Generator | 20–25 bar |

| Whistle/foghorn | 6–8 bar |

| Control Air | 6–7 bar (reduced through PRV) |

---

### Compressor Operating Procedure

#### Pre-Start Checks

1. Check LO level in compressor crankcase.

2. Check cooling water supply — ensure CW valves open.

3. Check inter-stage and final-stage drain valves (should be closed for normal start, open briefly to drain before start).

4. Verify air bottle drain valves are closed.

5. Check power supply, motor starter, isolator.

6. Confirm compressor is not isolated by maintenance.

#### Start Sequence

1. Open cooling water supply valve to compressor.

2. Close all drain valves.

3. Start compressor motor.

4. Observe inter-stage pressure: should build to 6–8 bar within 1–2 minutes.

5. Observe delivery temperature: should stabilise below 150°C.

6. Monitor LO pressure: minimum 1.5 bar.

7. Allow to run until bottle pressure reaches set point (usually 30 bar).

8. Compressor auto-stops at upper setpoint (auto-stop controller).

#### Draining Procedure (During Running)

Open drain valve briefly (5–10 seconds) every 30 minutes to remove water and oil condensate.

At each start: drain before charging to prevent condensate slugging.

If drain water appears oily: investigate oil carry-over; may indicate piston ring or valve wear.

Drain to oily water collecting tank; do NOT drain to bilge directly.

---

### Cross-Connection and Emergency Operations

#### Cross-Connection Valve

Links Air Bottle 1 to Air Bottle 2.

Normally CLOSED — kept separate for redundancy.

**Open** when:

- One bottle is empty/low — temporarily supply from the other.

- Testing cross-connection (periodic test per SMS).

- One bottle is isolated for maintenance.

**Close** after emergency use to restore separation.

#### Starting Air System Failure Response

1. Check bottle pressures — below 20 bar triggers alarm.

2. If ME start attempted with low air: starting failure will result.

3. Start both compressors immediately if low pressure.

4. Cross-connect bottles if one side low.

5. Check drain valves — if open, close and rebuild pressure.

6. If compressors fail: consult engine order/emergency procedure.

7. Log all events; report to master if ME cannot start.

---

### Safety Systems

#### Non-Return Valves (NRVs)

Fitted between compressor and air bottle — prevent reverse flow if bottle pressure > delivery.

Fitted between each air bottle and the main starting air line.

Fitted before each consumer connection.

#### Safety Relief Valves

| Location | Set Pressure |

|----------|-------------|

| Inter-stage separator | 9 bar |

| Final stage cooler | 32 bar |

| Air Bottle 1 | 32 bar |

| Air Bottle 2 | 32 bar |

#### Fusible Plug

Fitted in some air receivers as a thermal protection device.

Melts at 70–90°C to release air if receiver overheats (e.g., in a fire).

Do not reuse — replace after operation.

#### Oil/Water Separator (Moisture Separator)

Fitted after each compression stage and on compressor discharge line.

Removes oil droplets and water from compressed air.

Automatic drain trap or manual drain valve.

Blocked separator: high back pressure, reduced efficiency, oil carry-over risk.

---

### Auto-Start Logic (UMS — Unattended Machinery Spaces)

| Condition | Action |

|-----------|--------|

| Bottle pressure < 26 bar | Start duty compressor |

| Bottle pressure < 22 bar | Start standby compressor |

| Bottle pressure < 20 bar | Low pressure ALARM |

| Bottle pressure < 15 bar | Low-low pressure ALARM; me start not possible |

| Bottle pressure = 30 bar | Stop running compressor |

| Compressor fault | Alarm, start standby |

---

### Starting Air Contamination — Fire Risk

SOLAS requirement: Oil contamination of starting air systems is a significant fire hazard.

Oil/air mixture in high-pressure system can auto-ignite (diesel effect).

Keep crankcase oil level correct — overfilling leads to carry-over.

Change compressor oil at manufacturer's interval (typically 1000–2000 hours).

Clean oil separators and filters regularly.

If oil smell from drain: investigate immediately.

---

### Typical Starting Air Consumption

| Engine Type | Air Per Start | Starts Per Bottle (30 bar, 1m³) |

|-------------|--------------|--------------------------------|

| ME (4-stroke reversible) | ~0.6–1.5 m³ at 25 bar | 8–15 starts |

| ME (2-stroke direct) | ~1.5–3.0 m³ at 25 bar | 5–8 starts |

| AE/DG | ~0.3–0.6 m³ at 20 bar | 15–25 starts |

---

### References

SOLAS Chapter II-1, Regulation 26 — Starting Arrangements for Main Propulsion Machinery

IMO MSC/Circ.647 — Guidance on Maintenance of the Starting Air System

IACS UR M3 (2016) — Starting Air Systems for Main Propulsion Diesel Engines

DNV GL Rules for Classification, Ship Part 4 Chapter 3 — Rotating Machinery

Sperre AS: HV Compressor Series — Operation and Maintenance Manual

Hamworthy/Hatlapa: Air Compressor Manual

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System