BlueSim Training Guide

Dry Bulk Carrier Simulator

Cargo Simulators

Capesize 180,000 DWT — IMSBC Code 2022, BLU Code, SOLAS XII. 9 cargo holds with liquefaction risk monitoring, gas detection, hatch operations, and live stability calculation.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Capesize 180,000 DWT — IMSBC Code 2022, BLU Code, SOLAS XII. 9 cargo holds with liquefaction risk monitoring, gas detection, hatch operations, and live stability calculation.

## Key Features

6 cargo types: iron ore, coal, grain, bauxite, cement, fertilizer.

Live GM, trim, list, bending moment, and CH₄/CO gas detection per hold.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| STABILITY — TRIM · LIST · GM | Displays real-time system data, controls, and status indicators. |
| DECK PLAN — 9 CARGO HOLDS — CAPESIZE 180,000 DWT | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT SYSTEM | Displays real-time system data, controls, and status indicators. |
| CARGO HOLDS — 9 HOLDS — HATCH STATUS | Displays real-time system data, controls, and status indicators. |
| SCENARIO / TREND — DRAFT AFT | Displays real-time system data, controls, and status indicators. |
| HOLD MONITORING — GAS / MOISTURE / BILGE | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

OPEN ALL

CLOSE ALL

CRANE

${p.id}

DB PORT

DB STBD

TS PORT

TS STBD

H${h.id} VENT

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| LIQRISK | ALARM | Moisture > TML — LIQUEFACTION RISK — CEASE LOADING |
| CH4_LEL | ALARM | CH₄ > 20% LEL — GAS DANGER — RESTRICT ACCESS |
| CO_HIGH | WARNING | CO > 50 ppm — HOLD ATMOSPHERE HAZARDOUS |
| HATCH_LEAK | ALARM | Hatch cover leak — hold flooding detected |
| HOLD_FLOOD | ALARM | Hold bilge high level — investigate immediately |
| STRUCT_STRESS | ALARM | Bending moment > 90% limit — structural alarm |
| TRIM_HIGH | WARNING | Trim > 2.0m — check loading sequence |
| LIST_HIGH | ALARM | List > 3° — stability alarm — ballast required |
| GM_LOW | ALARM | GM < 0.15m — MINIMUM STABILITY REACHED |
| ESD_FIRED | ALARM | EMERGENCY SHUTDOWN ACTIVATED |
| LINEUP_FAULT | H | PUMP LINEUP FAULT — Pipeline incorrectly lined up |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| BT1_IV | No.1 Ballast Tank Isolation |
| BT2_IV | No.2 Ballast Tank Isolation |
| BT3_IV | No.3 Ballast Tank Isolation |
| DB_P_IV | Port Double Bottom Isolation |
| DB_S_IV | Stbd Double Bottom Isolation |
| SEA_CHEST | Sea Chest (High/Low) Valve |
| BWTS_IV | BWTS Bypass/Treatment Valve |
| BP1_SV | Ballast Pump 1 Suction Valve |
| BP1_DV | Ballast Pump 1 Discharge Valve |
| BP2_SV | Ballast Pump 2 Suction Valve |
| BP2_DV | Ballast Pump 2 Discharge Valve |
| DIST_XV | Distribution Crossover Valve |
| OVRBD | Overboard Discharge Valve |
| BILGE_SV | Bilge Pump Suction Valve |
| BILGE_OSD | Oil/Sludge Discharge Valve |
| FRESHWT | Fresh Water Tank Inlet Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL LOADING — IRON ORE

Sequenced loading maintaining stability per loading plan. Monitoring trim, GM, bending moments.

### Actions Required

Step 1: Verify hold inspection complete pre-loading

Step 2: Confirm loading sequence matches plan

Step 3: Monitor moisture content vs TML

Step 4: Check hatch weathertight seals

Step 5: Log draft readings each 1000 tonnes

## 5.2 LIQUEFACTION RISK — GROUP A CARGO

Moisture content rising toward TML. Cargo showing flow state characteristics. Assess and decide.

### Actions Required

Step 1: Stop loading — obtain additional moisture samples

Step 2: Request independent surveyor for TML test

Step 3: Notify Flag State / Classification Society

Step 4: If moisture > TML — refuse cargo

Step 5: Consider return to loading port

## 5.3 COAL GAS ACCUMULATION

CH₄ buildup in Hold 4. CO detected. Forced ventilation required. Restrict access.

### Actions Required

Step 1: Acknowledge CH₄ / CO alarms Hold 4

Step 2: Activate forced ventilation Hold 4

Step 3: Post gas-danger signs — restrict access

Step 4: Continuous monitoring — 4-hourly intervals

Step 5: If CH₄ > 20% LEL — cease all hot work

## 5.4 HATCH COVER LEAK — HOLD 6 FLOODING

Hatch 6 weathertight seal failure. Hold flooding detected. Activate bilge pumps.

### Actions Required

Step 1: Acknowledge bilge high-level alarm Hold 6

Step 2: Start bilge pump Hold 6

Step 3: Inspect hatch cover seals — gasket damage

Step 4: Apply emergency sealing compound

Step 5: Assess cargo damage — notify P&I Club

## 5.5 STABILITY ALARM — EXCESSIVE LIST

Cargo shift. List increasing. GM approaching minimum 0.15m. Emergency ballasting.

### Actions Required

Step 1: Acknowledge stability alarm

Step 2: Reduce speed — reduce wave action

Step 3: Ballast stbd tanks to correct list

Step 4: Monitor GM every 30 minutes

Step 5: Enter nearest port if GM < 0.15m

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System