BlueSim Training Guide

Chemical Tanker Simulator

Cargo Simulators

20-tank chemical tanker — IBC Code 2021, MARPOL Annex II. Multi-cargo operations with N₂ blanket, cargo heating, Butterworth tank cleaning, and IBC segregation compliance.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

20-tank chemical tanker — IBC Code 2021, MARPOL Annex II. Multi-cargo operations with N₂ blanket, cargo heating, Butterworth tank cleaning, and IBC segregation compliance.

## Key Features

6 selectable cargoes: methanol, styrene, caustic soda, ethanol, benzene, palm oil.

Emergency: polymerisation risk, N₂ failure, heating failure, overflow.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CARGO SYSTEM — GAUGES | Displays real-time system data, controls, and status indicators. |
| P&ID — CHEMICAL TANKER TANK ELEVATION — 20,000 DWT / 20 TANKS | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT SYSTEM | Displays real-time system data, controls, and status indicators. |
| TANK LEVELS — 20 TANKS (4 DISPLAYED) | Displays real-time system data, controls, and status indicators. |
| SCENARIO / TREND — CARGO TEMP | Displays real-time system data, controls, and status indicators. |
| VAPOUR SPACE / O₂ MONITORING | Displays real-time system data, controls, and status indicators. |
| PORT MANIFOLD | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

${p.id}

PORT MANIFOLD

STBD MANIFOLD

CROSSOVER

N₂ SUPPLY

N₂ HEADER

N₂ PURGE

STEAM SUPPLY

HEATER

COOLER

VENT MAST

P/V 1

P/V 2

P/V 3

EDUCTOR 1

EDUCTOR 2

STRIP P

STRIP S

DRAIN

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| TANK_OVERFLOW | ALARM | Tank level > 95% — CARGO OVERFLOW RISK |
| HIGH_TEMP_POLY | ALARM | Styrene temp > 40°C — POLYMERIZATION RISK |
| O2_INGRESS | ALARM | O₂ in vapour space > 5% — INERT ATMOSPHERE LOST |
| PUMP_CAVIT | WARNING | Deepwell pump cavitation — low suction |
| N2_FAIL | ALARM | N₂ supply failure — blanket integrity lost |
| HIGH_VAPOUR_PRESS | WARNING | High vapour pressure — check P/V valve |
| INCOMPAT_CARGO | WARNING | IBC Table 15 — Adjacent incompatible cargoes |
| LOW_TEMP_CAUSTIC | ALARM | Caustic soda temp < 20°C — solidification risk |
| HEAT_FAIL | ALARM | Cargo heating system failure |
| ESD_FIRED | ALARM | EMERGENCY SHUTDOWN ACTIVATED |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| T1_SV | Tank 1 Deepwell Suction Valve |
| T1_DV | Tank 1 Pump Discharge Valve |
| T1_SEG | Tank 1 Segregation Valve (NO) |
| T2_SV | Tank 2 Deepwell Suction Valve |
| T2_DV | Tank 2 Pump Discharge Valve |
| T2_SEG | Tank 2 Segregation Valve (NO) |
| T3_SV | Tank 3 Deepwell Suction Valve |
| T3_DV | Tank 3 Pump Discharge Valve |
| T3_SEG | Tank 3 Segregation Valve (NO) |
| T4_SV | Tank 4 Deepwell Suction Valve |
| T4_DV | Tank 4 Pump Discharge Valve |
| T4_SEG | Tank 4 Segregation Valve (NO) |
| MAN_1 | Manifold Valve No.1 (Stbd) |
| MAN_2 | Manifold Valve No.2 (Stbd) |
| MAN_P1 | Port Manifold Valve No.1 |
| STRIP_T1 | Tank 1 Stripping Valve |
| STRIP_T2 | Tank 2 Stripping Valve |
| STRIP_T3 | Tank 3 Stripping Valve |
| N2_PAD | N\u2082 Padding Supply Valve |
| VENT_IV | Vent Mast Isolation Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL MULTI-CARGO LOADING

All systems nominal. Monitoring cargo temperatures, N₂ blanket pressures and segregation compliance per IBC Code Table 15.

### Actions Required

Step 1: Verify N₂ blanket pressure 0.05–0.20 bar

Step 2: Confirm cargo heating at target temperature

Step 3: Check O₂ in vapour space < 2%

Step 4: Monitor pump discharge pressures

Step 5: Log cargo flow rates t/h

## 5.2 CARGO OVERFLOW — STYRENE POLYMERIZATION RISK

Tank 3P overflow alarm. Temperature rising in styrene tank. Risk of exothermic polymerization reaction. ACTIVATE ESD and cooling.

### Actions Required

Step 1: Acknowledge high-level alarm Tank 3P

Step 2: Stop deepwell pump DWP-1

Step 3: Close isolation valve Tank 3P

Step 4: Activate cargo cooler

Step 5: Activate ESD if temp > 40°C

Step 6: Notify MFAG / cargo owner

## 5.3 N₂ BLANKET FAILURE

N₂ supply header failure. O₂ ingress to vapour space detected. Benzene flash risk. Isolate tanks and restore inert atmosphere.

### Actions Required

Step 1: Acknowledge N₂ supply failure alarm

Step 2: Close tank vent master valves

Step 3: Isolate benzene tanks immediately

Step 4: Switch to emergency N₂ cylinders

Step 5: Monitor O₂ levels — target < 2%

## 5.4 HEATING FAILURE — CAUSTIC SOLIDIFICATION RISK

Steam heating failure. Caustic soda cooling toward solidification point 12°C. Emergency heat restoration required.

### Actions Required

Step 1: Acknowledge heating failure alarm

Step 2: Switch to emergency electric heaters

Step 3: Monitor tank temperature continuously

Step 4: If temp < 15°C — stop all pump operations

Step 5: Notify terminal of delay

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System