BlueSim Training Guide

LNG Tanker Cargo Simulator

Cargo Simulators

Q-Flex 216,000 m³ membrane LNG tanker — IGC Code 2016, STCW V/1-2 compliant. Full 11-phase voyage with BOG management, reliquefaction, and DFDE propulsion.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Q-Flex 216,000 m³ membrane LNG tanker — IGC Code 2016, STCW V/1-2 compliant. Full 11-phase voyage with BOG management, reliquefaction, and DFDE propulsion.

## Key Features

Cool-down, gassing-up, loading, laden voyage, discharge, and warm-up.

Emergency scenarios: rollover, rapid BOG, reliq failure, cryogenic fire.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| T1_IV | No.1 Tank Dome Isolation |
| T2_IV | No.2 Tank Dome Isolation |
| T3_IV | No.3 Tank Dome Isolation |
| T4_IV | No.4 Tank Dome Isolation |
| CP1_DV | Cargo Pump 1 Discharge Valve |
| CP2_DV | Cargo Pump 2 Discharge Valve |
| CP3_DV | Cargo Pump 3 Discharge Valve |
| LH_XV | Liquid Header Crossover |
| MAN_P | Port Manifold Isolation Valve |
| MAN_S | Stbd Manifold Isolation Valve |
| VRL_MAN | Vapour Return Line Manifold |
| VL_XV | Vapour Line Crossover Valve |
| SP_HV | Spray Header Isolation Valve |
| STRIP_IV | Stripping Line Isolation |
| IG_DV | IG/N₂ Deck Main Valve |
| IG_XV | IG Crossover Valve |

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 LNG Tanker Reference

## LNG Tanker Cargo Simulator — Regulatory Reference

### IGC Code 2016 · STCW V/1-2 · SIGTTO · BOG Management · −162°C Cryogenic Operations

---

### 1. Regulatory Framework

| Regulation | Key Requirement |

|-----------|----------------|

| **IGC Code** (IMO MSC.391(95), 2016) | Construction, equipment & ops for all gas carriers |

| **SOLAS Ch. VII Part C** | Ships carrying liquefied gases in bulk |

| **STCW V/1-2** | Advanced training for liquefied gas tanker cargo ops |

| **SIGTTO Ship/Shore Safety Checklist** | Mandatory before every transfer operation |

| **ISGOTT** (where applicable) | Industry supplement for operational guidance |

| **MARPOL Annex I** | ORB / cargo record keeping |

| **BWM Convention** | D-2 standard ballast water |

---

### 2. LNG Tanker Types & Containment Systems

#### Membrane Tanks (GTT Mark III / NO96)

Non-self-supporting — loads transmitted through insulation to inner hull

Primary barrier: 0.7mm corrugated stainless steel (Mark III) or Invar alloy (NO96)

Insulation: polyurethane foam + reinforced panels

Secondary barrier: integrated into insulation system

BOG: 0.10–0.15%/day typical; GTT NEXT1 (2024) improved to ~0.085%/day

Operating pressure: atmospheric ± small positive

Typical capacity: 125,000–216,000 m³ (Q-Flex/Q-Max up to 266,000 m³)

#### Moss Spherical (Type B)

Self-supporting aluminium spheres on cylindrical skirt

Independent from hull — no contribution to hull strength

BOG: ~0.12%/day

Weather cover above deck; spray shield at equator

Typical capacity: 125,000–138,000 m³

#### Key Material Requirements (IGC Code)

LNG at −162°C requires: **Invar (36% nickel)**, **stainless steel 304L/316L**, or **aluminium 5083/5086**

Carbon steel: NOT PERMITTED for any surface in contact with LNG

Secondary barrier material must withstand −163°C

---

### 3. BOG Management — IGC Code Requirement

IGC Code requires TWO independent means of managing excess vapour.

| Method | Description | When Used |

|--------|-------------|----------|

| **Reliquefaction plant** | N₂ refrigerant compresses BOG → condenses → returns to tank | Primary — laden passage |

| **GCU (Gas Combustion Unit)** | Burns BOG as waste heat | When BOG > reliq capacity, or reliq fails |

| **DFDE engine fuel** | Burns BOG in dual-fuel diesel-electric engines | During propulsion operations |

| **LD compressor → boiler** | Steam turbine plants: BOG to boilers | Traditional steam plants |

| **Vent mast** | Emergency release only | Last resort — no other option |

BOG rates:

Normal laden: 0.07–0.15% cargo volume/day

High sea state / high temp: up to 0.20%/day

Reliq plant designed for: 0.15%/day + safety margin

If BOG exceeds reliq capacity: GCU automatically activated

Condensate Return Plan:

Reliquefied LNG returned to cargo tanks via condensate return line (bottom spray)

Must monitor tank levels — reliq can slowly fill a tank beyond maximum

Tank level must not exceed operational maximum (typically 98.5%)

---

### 4. Full Lifecycle — Dry Dock to Dry Dock

#### Phase 1: Dry Dock

Cold-spot inspection: all hold spaces, cofferdams, interbarrier spaces

Class requirement: once per year minimum — divide into zones

Cargo pump overhaul (deepwell submersible)

Compressor shaft seal inspection — N₂ pressurisation system

Reliquefaction plant overhaul

IGC Certificate of Fitness renewal

Any permanent deformation of membrane recorded and reported to class

#### Phase 2: Gas Freeing (before entry)

LNG cargo remaining (heel) must first be warmed and vented

HC reduced to < 1% LEL — portable detectors at 3 levels

O₂ raised to 20.8% — confirm natural air composition

Entry certificate issued by Master

SCBA on standby at every tank entry point

For man-entry: minimum 2 persons, rescue equipment at entry point

#### Phase 3: Inerting

IGC Code requirement: tanks must be in inert condition before gassing-up.

N₂ generator or inert gas generator (IGG) supplies inert gas

Purge until O₂ < 2% vol in all tanks and associated pipework

O₂ analyser calibrated daily during inerting operations

Record all O₂ readings in Cargo Record Book (CRB)

#### Phase 4: Gassing-Up

Introduction of LNG vapour to displace N₂

HD (High Duty) compressor circulates vapour from shore or heel

HC content must reach > 90% vol (vapour-rich condition)

Purpose: condition tanks for liquid LNG loading — prevent hydrate formation

Hydrate inhibitors (methanol) may be used if moisture suspected

#### Phase 5: Cool-Down

Critical procedure — membrane integrity depends on controlled cooling.

Spray pumps deliver LNG through dome spray nozzles

Cooling rate must not exceed **10°C per hour**

Exceeding 10°C/hr risks thermal stress fracture of membrane

Monitor 3 temperature points in each tank (top, middle, bottom)

Target: all tanks at −160°C minimum before commencing liquid loading

Cool-down takes approximately 24–48 hours for large LNGC

#### Phase 6: Loading Port

SIGTTO Ship/Shore Safety Checklist — 135-point checklist before any transfer.

Loading procedure:

Initial flow rate: low (to prevent thermal shock to loading arms)

BOG generated goes to: HD compressor → shore vapour header (or GCU)

Tank filling: lower tanks first (reduces sloshing risk)

Vapour return line: mandatory — shore must accept vapour

Maximum loading rate: ~12,000 m³/hr for 216,000 m³ LNGC

Stop loading 2% below 100% fill — sloshing damage risk

ESD System:

Ship and shore ESD linked before transfer commences

ESD test before every operation

Two independent ESD systems required (IGC Code)

ESD closes all cargo valves, stops pumps and compressors

ESD response time: ≤ 30 seconds

#### Phase 7: Laden Sea Passage

BOG management strategy determined by charter/operational economics

Cold-spot inspection: all interbarrier and hold spaces — every laden voyage

Brittle fracture risk: structural steel at low temperature — monitor cold spots

Condensate return plan: prevent tank overflow from reliquefied return

Rollover prevention: monitor cargo density stratification

- Rollover: two stratified layers mix spontaneously → rapid BOG generation

- Caused by: different cargo sources/densities, weathering, temperature gradients

- Prevention: circulate cargo between tanks, monitor density at top and bottom

4-hour watch gauging: all tanks, temperatures, pressures

#### Phase 8: Discharge Port

Deepwell submersible pumps: no cargo pump room (IGC Code — no cargo pumps below deck)

Vaporiser: converts LNG to gas for shore send-out piping

Heel management: retain minimum volume to keep tanks cold between voyages

- Typical heel: 0.5–2% of cargo capacity

- Purpose: avoids full cool-down requirement next voyage

Stripping: final removal of cargo — spray pump to lowest level

All CRB entries completed before departure

#### Phase 9–11: Warm-Up, Gas Freeing, Ballast

Same procedures as pre-loading but in reverse

If drydock: full gas free → inert → warm tanks to ambient

---

### 5. Propulsion Systems

| Type | Description | BOG Use |

|------|-------------|---------|

| **Steam turbine** | Traditional — two boilers | All BOG burned in boilers |

| **DFDE** | Dual-Fuel Diesel-Electric | BOG via LD compressor to DF engines |

| **ME-GI** | High-pressure gas injection diesel | BOG at 300 bar via HP compressor |

| **TFDE** | Tri-Fuel Diesel-Electric | BOG + HFO + MDO capability |

Methane slip: ME-GI has lowest slip (<0.1 g/kWh); DFDE higher (~4 g/kWh). FuelEU Maritime and CII penalise methane slip (GWP×25).

---

### 6. Emergency Procedures

#### Rollover

Signs: rapid pressure rise, temperature changes, unusual BOG generation

Actions: stop loading/discharge, activate GCU, circulate cargo between tanks

Prevention: homogeneous loading, avoid stratification

#### Reliquefaction Plant Failure

GCU activates automatically

Reduce speed if BOG generation from propulsion can absorb excess BOG

Check N₂ refrigerant supply

Contact class/maker for repair guidance

#### Cryogenic Fire

**DO NOT use water jet** — thermal shock risk to cryogenic structure

Dry powder or CO₂ for direct fire attack

Water curtain acceptable for boundary protection and cooling adjacent structure

BLEVE (Boiling Liquid Expanding Vapour Explosion) risk if fire impinges on tank

Evacuate area — large cold zone extends well beyond visible flame

#### Gas Detection in Hold Space

Indicates possible primary barrier leak

ESD standby

Increase N₂/IG pressure to interbarrier space

Reduce cargo pressure if possible

Contact class — do not continue cargo operations

---

### 7. Sources

1. IGC Code (IMO MSC.391(95), 2016 edition) — IGC Code

2. STCW V/1-2 — Advanced Training for Liquefied Gas Tanker Cargo Operations

3. SIGTTO — Ship/Shore Safety Checklist & LNG Operational Practice

4. GTT — Mark III containment system technical documents (2024 NEXT1 approval)

5. Wikipedia — Gas carrier (updated October 2025) — containment system types

6. sea-man.org — "Mastering Laden Voyage for Liquefied Gas Carriers" (June 2024)

7. cultofsea.com — Gas Tanker Types, Tanks, Reliquefaction (July 2019)

8. sea-man.org — LNG/LPG Reliquefaction plant systems (June 2024)

9. USCG CGTTP 3-72.6A — Foreign Gas Carrier Examiner guide

10. Lloyd's Register — Rules for Ships Carrying Liquefied Gases (July 2025)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System