BlueSim Training Guide

Methanol Cargo Tanker Simulator

Cargo Simulators

50,000 DWT methanol tanker — IBC Code (NLS Type II), MARPOL Annex II. Anti-static earthing, Coriolis metering, N₂ blanket, and flash point monitoring.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

50,000 DWT methanol tanker — IBC Code (NLS Type II), MARPOL Annex II. Anti-static earthing, Coriolis metering, N₂ blanket, and flash point monitoring.

## Key Features

12 deepwell pumps, flash point 11°C, vapour IDLH 6000 ppm.

Emergency: flash point alarm, N₂ failure, pump room accumulation.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CARGO SYSTEM — CH₃OH | Displays real-time system data, controls, and status indicators. |
| P&ID — METHANOL CARGO CONTAINMENT (IBC CODE) | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT — MARPOL/IBC | Displays real-time system data, controls, and status indicators. |
| GAS DETECTION — CH₃OH PPM (IDLH: 6000 ppm) | Displays real-time system data, controls, and status indicators. |
| SCENARIO / TREND | Displays real-time system data, controls, and status indicators. |
| CARGO METERING / PPE | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Avg Tank Level | % |
| Cargo Temp | °C |
| Flash Point | °C |
| Vapour Pressure | kPa |
| O₂ Vapour Space | % |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

N₂ SUPPLY: ON

P${i+1}

`

T${i+1}

${v.label}

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| FP_HIGH | ALARM | Flash point >15°C — cargo contamination suspect |
| O2_HIGH | ALARM | O₂ >2% in vapour space — inerting loss |
| METHANOL_200 | WARNING | Methanol vapour >200 ppm TLV-TWA — crew exposure |
| METHANOL_6000 | ALARM | IDLH 6000 ppm EXCEEDED — EVACUATE / SAR mandatory |
| OVERFLOW | ALARM | Cargo overflow — tank >98% — STOP TRANSFER |
| PUMP_FAIL | ALARM | Deepwell pump failure — no flow response |
| N2_LO_PRESS | ALARM | N₂ supply pressure low — blanket at risk |
| ANTISTATIC | ALARM | Anti-static earthing fault — manifold connection |
| ESD_FIRED | ALARM | EMERGENCY STOP TRANSFER ACTIVATED |
| BLANKET_LO | WARNING | N₂ blanket pressure low — vapour space unprotected |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| T1_IV | Cargo Tank 1 Isolation Valve |
| T2_IV | Cargo Tank 2 Isolation Valve |
| T3_IV | Cargo Tank 3 Isolation Valve |
| T4_IV | Cargo Tank 4 Isolation Valve |
| CP1_SV | Cargo Pump 1 Suction Valve |
| CP1_DV | Cargo Pump 1 Discharge Valve |
| CP2_SV | Cargo Pump 2 Suction Valve |
| CP2_DV | Cargo Pump 2 Discharge Valve |
| CX_1 | Pipeline Crossover Valve 1 |
| MAN_P | Port Manifold Isolation |
| MAN_S | Stbd Manifold Isolation |
| VENT_IV | Cargo Tank Vent Line Isolation |
| N2_PAD | N\u2082 Blanket/Padding Valve |
| STRIP_IV | Stripping Line Isolation Valve |
| HOT_COIL | Heating Coil Isolation Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL LOADING OPERATIONS

Steady-state CH₃OH loading per IBC Code. Monitor FP, N₂ blanket, anti-static earthing.

### Actions Required

Step 1: Verify cargo temp 15–25°C

Step 2: Confirm N₂ blanket pressure >0

Step 3: Check all earthing continuity

Step 4: Monitor O₂ in vapour space <2%

Step 5: Log Coriolis meter reading

## 5.2 FLASH POINT ALARM — CONTAMINATION

Water contamination raises FP above 15°C. Identify contaminated tank. Notify flag state under MARPOL Annex II.

### Actions Required

Step 1: Acknowledge FP alarm

Step 2: Stop loading to contaminated tank

Step 3: Collect cargo sample for analysis

Step 4: Notify terminal and flag state

Step 5: Isolate contaminated tank valve

## 5.3 N₂ BLANKET FAILURE

N₂ supply fails. O₂ ingress to vapour space. Flammable atmosphere risk above 4% O₂.

### Actions Required

Step 1: Acknowledge N₂ pressure alarm

Step 2: Switch to backup N₂ supply

Step 3: Suspend cargo transfer

Step 4: Monitor O₂ continuously

Step 5: Activate vent mast HV valves

## 5.4 MANIFOLD VAPOUR LEAK

Flange leak at port manifold. Methanol vapour release. PPM rising. Stop transfer and evacuate manifold area.

### Actions Required

Step 1: Acknowledge manifold vapour alarm

Step 2: Activate ESD — stop transfer

Step 3: Don SAR / chemical suit

Step 4: Close manifold valves P/S

Step 5: Apply decontamination

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System