BlueSim Training Guide

Very Large Ethane Carrier (VLEC) Simulator

Cargo Simulators

99,000 m³ VLEC with 4 bilobe tanks at −104°C — IGC Code Ch.19, STCW V/1-2, SIGTTO. Carries ethane and ethylene with full BOG management, reliquefaction, and cool-down operations.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

99,000 m³ VLEC with 4 bilobe tanks at −104°C — IGC Code Ch.19, STCW V/1-2, SIGTTO. Carries ethane and ethylene with full BOG management, reliquefaction, and cool-down operations.

## Key Features

Bilobe P&ID canvas, BOG rate vs reliq duty, sub-cooling margin, ethylene mode toggle.

Emergency: BOG excess, reliq overload, manifold gas leak, cargo overflow.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CARGO SYSTEM — C₂H₆ / C₂H₄ | Displays real-time system data, controls, and status indicators. |
| P&ID — VLEC 99,000 m³ BILOBE CARGO CONTAINMENT SYSTEM | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT — ESD CONTROLS | Displays real-time system data, controls, and status indicators. |
| GAS DETECTION — % LEL (LFL 3.0% · UFL 12.4%) | Displays real-time system data, controls, and status indicators. |
| SCENARIO / TREND | Displays real-time system data, controls, and status indicators. |
| COOL-DOWN MONITOR · CARGO MODE | Displays real-time system data, controls, and status indicators. |
| HD-C1 | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Tank Temp (avg) | °C |
| Tank Pressure | barg |
| Cargo Flow | t/h |
| BOG Rate | %/day |
| Reliq Duty | % |
| Sub-cool Margin | °C |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

TANK SPRAY

N₂ PURGE

PORT

STBD

T${i+1}

XOVER

BOG RTN

EMER VENT

ESD M/V

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| TANK_HI_PRESS_W | WARNING | Tank pressure high — > 0.25 barg |
| TANK_HI_PRESS_A | ALARM | Tank pressure HIGH ALARM — > 0.35 barg |
| TANK_HI_TEMP | WARNING | Tank temperature deviation — > −99°C |
| TANK_LO_LEVEL | WARNING | Tank level low — < 5% |
| BOG_EXCESS | WARNING | BOG rate excess — > 0.25% per day |
| LEL_10 | WARNING | Gas concentration > 10% LEL WARNING |
| LEL_20 | ALARM | Gas concentration > 20% LEL ACTION — EVACUATE |
| O2_LOW | ALARM | O₂ depletion — < 19.5% — NO ENTRY |
| RELIQ_FAIL | ALARM | Reliq overload — compressor duty > 95% |
| ESD_FIRED | ALARM | EMERGENCY SHUTDOWN ACTIVATED — ALL OPS SUSPENDED |
| COOLDOWN_RATE | WARNING | Cool-down rate deviation — > 3°C/h from target |
| CARGO_OVERFLOW | ALARM | CARGO OVERFLOW RISK — tank level > 98% |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| T1_IV | No.1 Tank Dome Isolation Valve |
| T2_IV | No.2 Tank Dome Isolation Valve |
| T3_IV | No.3 Tank Dome Isolation Valve |
| T4_IV | No.4 Tank Dome Isolation Valve |
| CP1_DV | Cargo Pump 1 Discharge Valve |
| CP2_DV | Cargo Pump 2 Discharge Valve |
| LH_XV | Liquid Header Crossover Valve |
| MAN_P | Port Manifold Isolation |
| MAN_S | Stbd Manifold Isolation |
| VRL_MAN | Vapour Return Line Manifold |
| VL_XV | Vapour Line Crossover Valve |
| COOL_IV | Cooling Spray Isolation Valve |
| RELIQ_IN | Reliq Unit Suction Isolation |
| RELIQ_OUT | Reliq Unit Discharge Isolation |
| N2_IV | N\u2082 Hold Space Valve |
| STRIP_IV | Stripping Line Isolation |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL SEA PASSAGE

Laden voyage. Ethane at −104°C / 0.05 barg. HD compressors reliquefying BOG. Monitor all parameters per SIGTTO.

### Actions Required

Step 1: Confirm tank temp ≤ −99°C

Step 2: Confirm tank press ≤ 0.25 barg

Step 3: HD compressor 1 on — reliq active

Step 4: Log BOG rate (target 0.15%/day)

Step 5: Record all tank levels

## 5.2 BOG EXCESS — RELIQ OVERLOAD

Swell / warm weather causing excess BOG. Reliq approaching capacity limit. Tank pressure rising. Decision required.

### Actions Required

Step 1: Acknowledge BOG excess alarm

Step 2: Start HD compressor 2

Step 3: Reduce ship speed to reduce heat ingress

Step 4: Consider BOG return to engine (LD comp)

Step 5: Monitor tank pressure — limit 0.35 barg

## 5.3 COOL-DOWN PHASE

Pre-loading cool-down. Spray ethane liquid onto tank walls. Target −104°C. Max rate 3°C/h per IGC Code.

### Actions Required

Step 1: Activate tank spray system

Step 2: Verify rate ≤ 3°C/h

Step 3: Monitor structural steel temp diff

Step 4: Check tank pressure stays < 0.25 barg

Step 5: Confirm all 4 tanks at −104°C

## 5.4 GAS LEAK — CARGO MANIFOLD

Flange leak detected at manifold. C₂H₆ rising in manifold zone. Execute ESD sequence. Muster crew. IGC §19.8.

### Actions Required

Step 1: Acknowledge 10% LEL alarm

Step 2: Activate ESD — close manifold valves

Step 3: Stop all cargo pumps

Step 4: Muster crew — check accommodation

Step 5: Ventilate manifold area

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System