BlueSim Training Guide

LPG Fuel Ship Simulator

Fuel Simulators

LPG-fuelled ship operations — IGF Code and IMO LPG fuel guidelines, covering propane/butane fuel systems and reliquefaction.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

LPG-fuelled ship operations — IGF Code and IMO LPG fuel guidelines, covering propane/butane fuel systems and reliquefaction.

## Key Features

Fully refrigerated storage, cargo pump and compressor controls.

ESD and bunkering scenarios with alarm management.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Tank Pressure | bar |
| Tank Temp | °C |
| Tank Level | % |
| Fuel Flow | t/h |
| Vaporiser T | °C |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

`

${v.label}

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| LEL10_FP | WARNING | LPG 10% LEL — Fuel Prep Room |
| LEL50_FP | F | LPG 50% LEL — Fuel Prep Room — ESD |
| LEL10_BI | WARNING | LPG 10% LEL — BILGE (gas sinking) — purge bilge |
| LEL50_BI | F | LPG 50% LEL — BILGE — explosion risk — ESD |
| LEL10_ER | WARNING | LPG 10% LEL — Engine Room floor level |
| LEL50_ER | F | LPG 50% LEL — Engine Room — evacuate |
| LEL10_DK | WARNING | LPG 10% LEL — Deck / Bunker area |
| PRESS_HI | WARNING | Tank pressure high — check temperature / PRV |
| PRESS_LO | WARNING | Tank pressure low — check vaporiser supply |
| ESD_FIRED | F | EMERGENCY SHUTDOWN ACTIVATED |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| TANK_IV1 | LPG Tank Primary Isolation Valve |
| TANK_IV2 | LPG Tank Secondary Isolation Valve |
| PUMP_DV | Fuel Pump Discharge Valve |
| VAPORISER_IV | LPG Vaporiser Inlet Isolation |
| VAPORISER_OV | LPG Vaporiser Outlet Isolation |
| MFV | Master Fuel Valve (gas side) |
| DBB1 | Double Block & Bleed — Block 1 |
| DBB2 | Double Block & Bleed — Block 2 |
| DBB_BLEED | Double Block & Bleed — Bleed Valve |
| GVU_MAIN | Gas Valve Unit — Main Gas Valve |
| GVU_ESD | GVU Emergency Shutdown Valve |
| RELIQ_SUCT | Reliquefaction Unit Suction |
| RELIQ_DISC | Reliquefaction Unit Discharge |
| BUNK_IV | Bunker Line Isolation Valve |
| BUNK_DBB | Bunker Double Block Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL OPERATIONS

LPG fuel system steady state. ~8 bar pressurised storage. Vaporiser converting liquid to gas. LPG heavier than air — bilge detectors key.

### Actions Required

Step 1: Verify all LEL detectors < 5% (floor level)

Step 2: Check tank pressure 7–9 bar

Step 3: Confirm vaporiser online

Step 4: Log fuel consumption t/h

## 5.2 LPG LEAK — BILGE ACCUMULATION

Propane/butane is heavier than air. A small leak accumulates in bilges silently. Bilge detector alert first. ESD before LEL 50%.

### Actions Required

Step 1: Acknowledge bilge LEL 10% alarm

Step 2: Close supply valve — isolate leak source

Step 3: Purge bilge with forced ventilation

Step 4: Activate ESD if LEL reaches 50%

Step 5: Confirm no ignition sources in bilge

## 5.3 LPG LIQUID LINE FAULT

LPG bleed line releasing liquid. Flash vaporisation creates rapid LEL rise. Liquid LPG = much more dangerous than gas.

### Actions Required

Step 1: Close supply valve immediately

Step 2: Acknowledge multi-zone LEL alarm

Step 3: Activate ESD

Step 4: Remove all ignition sources

Step 5: Water spray — dilute vapour cloud

## 5.4 LPG BUNKERING / CARGO-AS-FUEL

Transferring LPG from shore or from ship cargo tanks. Vapour return mandatory. Purge with N₂ before disconnect.

### Actions Required

Step 1: Connect vapour return line first

Step 2: ESD compatibility test completed

Step 3: Purge line with N₂ before opening LPG

Step 4: Open bunker valve slowly — confirm flow

Step 5: Monitor bilge and deck LEL detectors

Step 6: Close valve — N₂ purge before disconnect

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 LPG Regulatory Reference

## LPG Fuel Ship Simulator — Regulatory Reference & Training Guide

### IMO MSC.1/Circ.1666 · Interim Guidelines for Safety of Ships Using LPG Fuels

Document: MSC.1/Circ.1666

Approved: MSC 107, 31 May–9 June 2023

Additional: MSC.1/Circ.1679 (LPG cargo as fuel, approved MSC 108 May 2024)

Basis: IGF Code Parts B-1, C-1, D apply unless expressly stated otherwise

Ships on order: 139 (LR/IHS/Clarksons, Jan 2026) — mainly LPG carriers using cargo as fuel

---

### 1. LPG Properties — The Critical Difference from LNG/Methanol/Ammonia

LPG is HEAVIER THAN AIR — the single most important training difference.

| Property | LPG (C₃H₈/C₄H₁₀) | LNG (CH₄) | Methanol (CH₃OH) |

|----------|-------------------|-----------|-----------------|

| Vapour density vs air | **1.5–2.0 (HEAVIER)** | 0.55 (lighter) | 1.1 (slightly heavier) |

| Gas accumulation | Sinks — bilges, pits, low spaces | Rises — ceilings | Near floor/mid |

| Detector placement | **FLOOR LEVEL** | Ceiling level | Floor + mid |

| Storage condition | Pressurised (~8 bar) OR refrigerated (−42°C) | Cryogenic (−162°C) | Ambient |

| LEL (propane) | **2.1%** | 5% | 6% |

| LEL (butane) | **1.8%** | — | — |

| UEL (propane) | 9.5% | 15% | 36.5% |

| Flashpoint | −104°C (propane) | −188°C | 11°C |

| Flame visibility | Visible | Visible | **Invisible** |

| Primary hazard | Fire/explosion in low spaces | Cryogenic + fire | Fire + invisible flame |

#### Why Floor-Level Detection Matters

LPG is significantly denser than air. A small propane leak in an enclosed space will not mix evenly — it sinks and pools at the lowest point. In a machinery space, this means:

Bilges accumulate explosive concentrations while mid-space reads zero

The crew may be unaware of a dangerous bilge condition

A spark at bilge level (pump starting, tools dropping) can ignite the accumulated gas

Training Rule: Always check bilge LEL reading before entering any space that may have had LPG exposure. Never trust only the mid-level detector.

---

### 2. LPG Composition

LPG in marine fuel applications is a blend of:

**Propane (C₃H₈):** 40–80% by volume — boiling point −42°C

**Butane (C₄H₁₀):** 20–60% by volume — boiling point −1°C

Small quantities of ethane, pentane, and impurities

The exact ratio affects:

Operating pressure (higher propane = lower boiling point = higher vapour pressure at ambient temp)

LEL value (propane LEL 2.1% is lower than butane LEL 1.8% — propane is more hazardous)

Cold-climate performance (pure butane gels at very low temperatures)

---

### 3. Storage Systems (per MSC.1/Circ.1666 §5)

#### Type C Pressurised (Most Common for LPG Fuel)

Horizontal cylindrical pressure vessel

Typically 7–10 bar at ambient temperature

No cryogenic systems required — simpler than LNG

PRV (Pressure Relief Valve) mandatory

No boil-off issue — LPG stays liquid under pressure at ambient temperature

#### Refrigerated Storage (−42°C, atmospheric)

Used on larger LPG carriers using cargo as fuel

Similar to LNG but at much higher temperature (−42°C vs −162°C)

Less PPE complexity than LNG — no extreme cryogenic cold burn risk

Requires small reliquefaction capacity to manage heat ingress

#### Key MSC.1/Circ.1666 Tank Requirements

LPG liquid lines in enclosed spaces: **double-walled piping** (per IGF Code alignment)

LPG liquid line bleed connections: must route to tank or gas-liquid separator — **NOT to atmosphere**

Tank: PRV mandatory; PRV discharge to safe location

ESD-protected machinery spaces: may be permitted (alternative design under SOLAS II-1/55)

---

### 4. LEL Detection Thresholds

| % LEL | Propane Conc. | Action |

|-------|--------------|--------|

| 10% LEL | 2,100 ppm (0.21% vol) | Warning — investigate source |

| 50% LEL | 10,500 ppm (1.05% vol) | ALARM — ESD, evacuate |

| 100% LEL | 21,000 ppm (2.1% vol) | At LEL — imminent explosion |

Detector technology: Catalytic bead (pellistor) suitable for both propane and butane. IR sensors also work well. Placement: bilge, floor corners of machinery spaces, fuel prep room low level.

---

### 5. Vaporiser Unit

Unlike LNG which has BOG management, LPG fuel systems use a vaporiser to convert liquid LPG to gas for engine consumption:

Liquid LPG from tank → Vaporiser (electric or seawater heated) → Gas supply to engine

Vaporiser must maintain stable outlet temperature and pressure

Vaporiser fault = fuel gas supply disruption = engine trip to diesel

Temperature monitoring on vaporiser outlet is critical

---

### 6. ESD System (per MSC.1/Circ.1666, aligned with IGF Code §15)

ESD triggers:

Manual activation (bridge, ECR, fuel prep room)

LEL ≥ 50% detection (automatic)

Loss of ventilation in fuel prep room

Vaporiser fault causing gas overpressure

Power failure (fail-safe closed)

ESD cascade:

1. Master Fuel Valve (MFV) closes

2. Supply valve closes

3. Vaporiser trips

4. Engine switches to diesel/HFO mode

5. Emergency ventilation activates

---

### 7. Training Scenarios

#### Scenario 1 — Normal Operations

Normal operating parameters:

Tank pressure: 7–9 bar (ambient temperature Type C)

Tank temperature: −5 to +30°C (propane vapour pressure varies)

LEL all zones: < 5% (especially bilge)

Vaporiser outlet: 40–60°C

#### Scenario 2 — Bilge Accumulation

LPG sinks — bilge detector rises while crew may not notice any odour at nose/chest level.

1. Check bilge LEL when alarm activates — do NOT dismiss

2. Close supply valve — isolate source

3. Force-ventilate bilge space (ventilation inlet at TOP, outlet at BOTTOM)

4. Remove all ignition sources before entering

5. ESD if LEL ≥ 50%

Key training point: Propane has an odour at concentrations > 1,000 ppm. However, olfactory fatigue can occur, and 1,000 ppm = 4.8% LEL — already nearly half the warning threshold. Never rely on smell alone.

#### Scenario 3 — Liquid Line Release

LPG liquid release is far more dangerous than gas release:

1 litre of liquid LPG = approximately 270 litres of vapour

Flash vaporisation creates rapid LEL buildup

Liquid LPG contacting skin = cold burn at −42°C (propane)

Response:

1. Close valves immediately — stop the source

2. ESD activation

3. Water spray to dilute/disperse vapour cloud

4. Do NOT use water jet directly on liquid LPG (spreads it)

5. No ignition sources

#### Scenario 4 — LPG Bunkering

Procedure:

1. Connect vapour return line first

2. ESD compatibility test

3. Purge bunker line with N₂ (dry nitrogen) before opening LPG valve

4. Open LPG valve slowly

5. Monitor bilge and deck LEL throughout transfer

6. On completion: close valve → N₂ purge → confirm line pressure zero → disconnect

LPG vs LNG bunkering: No cool-down required (ambient temperature), but N₂ purging is mandatory. The critical risk is LPG liquid in the hose — it must be fully vapourised and purged before disconnect.

---

### 8. Comparison with Other Simulators

| Feature | LPG | LNG | CH₃OH | NH₃ |

|---------|-----|-----|-------|-----|

| IMO Standard | MSC.1/Circ.1666 (2023) | IGF Code (2017) | MSC.1/Circ.1621 (2020) | MSC.1/Circ.1687 (2024) |

| Storage | 8 bar pressurised | −162°C cryo | Ambient | −33°C cryo |

| Key hazard | Heavier-than-air fire | Cryo burns + fire | Invisible fire | Toxicity |

| Detector placement | **FLOOR** | Ceiling | Floor | Any level |

| Unique system | Vaporiser | BOG + reliq | N₂ inerting | Reliquefaction |

| Cool-down needed | No | **Yes** | No | No |

| BOG management | No (pressurised) | **Yes** | No | Yes |

---

### 9. Sources

1. IMO MSC.1/Circ.1666 — Interim Guidelines for Safety of Ships Using LPG Fuels (June 2023)

2. IMO MSC.1/Circ.1679 — Interim Guidelines for Ships Using LPG Cargo as Fuel (May 2024)

3. IBIA — "IMO approves Interim LPG Guidelines" (June 2023)

4. ABS Regulatory News MSC 107 — CCC 11 briefing (June 2023). eagle.org

5. IMO.org — MSC 107th session summary (2023). imo.org

6. DNV — Alternative Fuels Insight platform — LPG data. dnv.com

7. Lloyd's Register — "Alternative-fuelled ship orders remain significant in 2025" (Jan 2026). lr.org

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System