BlueSim Training Guide

LNG Fuel Gas Supply System Simulator

Fuel Simulators

LNG FGSS simulator for dual-fuel vessels covering Type C tank management, gas supply train startup, BOG control and ESD response per IGF Code.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

LNG FGSS simulator for dual-fuel vessels covering Type C tank management, gas supply train startup, BOG control and ESD response per IGF Code.

## Key Features

Type C cryogenic LNG tank with pressure, level, temperature and BOG rate monitoring.

Full gas supply train: LNG pump, vaporiser, GVU with DBB valves, engine gas rail.

Gas detection in pump room, GVU room and engine room with ESD trip simulation.

Four training scenarios: gas mode startup, ESD trip, BOG management, pressure control.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| LNG TANK & SUPPLY | Displays real-time system data, controls, and status indicators. |
| FGSS PROCESS SCHEMATIC | Displays real-time system data, controls, and status indicators. |
| DETECTION & VALVES & ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| TANK PRESSURE TREND | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

VAP IN [C]

VAP OUT [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## LNG Fuel Gas Supply System (FGSS) — Technical Reference

### Regulatory Framework

**IGF Code (International Code of Safety for Ships using Gases or Other Low-Flashpoint Fuels)**: MSC.391(95) — primary regulation for LNG-fuelled vessels.

**IMO IGF Code Part A-2**: Requirements for methane (natural gas) as fuel.

**SOLAS II-1/Reg.55**: Requirements for ships using low-flashpoint fuels.

**MSC.1/Circ.1621**: Guidelines for approval of alternatives and equivalencies to the IGF Code.

**ISO 20519**: Specification for bunkering of LNG-fuelled vessels.

**Classification Rules**: DNV Rules for Classification — Ships / Part 6 Ch. 2 (Gas fuel systems).

### LNG Fuel Tank (Type C)

#### Type C Tank Characteristics

Cylindrical pressure vessel (independent tank, no structural hull connection)

Design pressure: 4–10 bar (typical for type C)

Minimum design temperature: −165°C

Hold space: double wall with vacuum/perlite insulation

Boil-Off Gas (BOG) rate: 0.1–0.2% of tank volume per day

Normal operating pressure: 2–7 bar (absolute)

#### Key Parameters

| Parameter | Normal Range | Alarm Low | Alarm High |

|-----------|-------------|-----------|------------|

| Tank pressure | 1.5–6.0 bar | < 0.8 bar | > 6.5 bar |

| Tank level | 10–95% | < 10% | > 98% |

| LNG temperature | −162°C to −155°C | — | > −140°C |

| BOG rate | 0.1–0.5 kg/h per ton | — | > 1.0 kg/h/ton |

### Gas Supply Train

#### Component Description

1. LNG Pump (Submerged): Cryogenic centrifugal pump inside LNG tank. Pumps LNG at high pressure to vaporiser. Typical pressure: 5–8 bar.

2. Vaporiser (Heat Exchanger): Converts LNG to natural gas. Types: sea water heated, glycol water heated. Outlet temp: +20°C to +40°C at full pressure.

3. GVU (Gas Valve Unit): Contains master gas valve, pressure regulating valves, flow meter, gas detection. Located in ventilated room.

4. Engine Gas Fuel Rail: Delivers gas at engine inlet pressure (typically 6–8 bar for MAN B&W or Wärtsilä DF engines).

#### Double Block and Bleed (DBB) Valves

Required by IGF Code at gas admission valves to each engine

Two shut-off valves in series with bleed valve between

On ESD: both block valves close; bleed vents gas between them to safe location

Ensures positive isolation of gas supply to engine

### BOG (Boil-Off Gas) Management

#### Sources of BOG

Heat ingress through tank insulation (continuous)

LNG pump operation (mechanical heat)

Pressure build-up during voyage (natural evaporation)

#### BOG Handling Options

1. Forced Vaporisation: Use BOG in engine as primary fuel (preferred)

2. BOG Compressor: Compress BOG to supply gas train pressure

3. Vent Mast: Emergency release of excess BOG to atmosphere (last resort)

4. Re-liquefaction: Cryogenic compressor (large vessels only)

### Operating Modes

**GAS MODE**: LNG pump running, gas supply to engine active. Engine burning gas + pilot fuel.

**PILOT OIL / HFO**: Gas system offline. Engine running on diesel/HFO.

**DUAL FUEL**: Both gas and pilot oil injected (during transition).

### ESD (Emergency Shutdown) System

#### ESD Triggers

Gas leakage detected (>20% LEL in GVU room or ER)

Engine room fire

Manual ESD activation

Loss of gas pressure

LNG tank high pressure

#### ESD Actions

1. Close master gas valve (MGV)

2. Close all engine gas inlet valves (DBB close)

3. Stop LNG pump

4. Isolate vaporiser

5. Switch engine to pilot oil/HFO mode

6. Activate gas dispersion system

### Gas Detection

Catalytic bead sensors (0–100% LEL) in engine room, GVU room, LNG pump room

Alarm at 20% LEL

ESD at 40% LEL (automatic)

Infrared sensors for continuous monitoring

### Fuel Mode Indicators

**GAS**: Primary combustion gas; pilot oil ≤ 1% of fuel energy

**PILOT OIL**: Diesel only; gas system isolated

**HFO**: Heavy fuel oil mode (if applicable)

### Key Calculations

**BOG rate (kg/h)** = 0.001 × tank_volume(m³) × LNG_density(kg/m³) × BOG_rate(%/day) / 24

**Gas consumption** ≈ 0.135–0.165 kg/kWh (LHV basis for DF engines)

**LNG density at −162°C**: approx. 425–450 kg/m³

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System