BlueSim Training Guide

SCR (Selective Catalytic Reduction) Simulator

Engine Room Equipment

Wärtsilä/MAN SCR system for NOx reduction using urea (AdBlue) per MARPOL Annex VI Reg.13 Tier III and NTC 2008.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Wärtsilä/MAN SCR system for NOx reduction using urea (AdBlue) per MARPOL Annex VI Reg.13 Tier III and NTC 2008.

## Key Features

SCR with urea injection system — NOx reduction up to 90%, achieving Tier III 3.4 g/kWh.

Catalyst temperature monitoring with low-temperature bypass interlock (min 320°C).

Ammonia slip monitoring — IACS UR M74 limit of 10 ppm enforced with alarm.

Catalyst preheating sequence with real-time readiness indicator.

Scenarios: Tier III Activation, Low Temperature Bypass, Ammonia Slip Alarm, Urea Tank Low.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| SCR CONTROLS | Displays real-time system data, controls, and status indicators. |
| P&amp;ID — SCR SYSTEM &amp; NOx REDUCTION GRAPH | Displays real-time system data, controls, and status indicators. |
| GAUGES &amp; ALARMS | Displays real-time system data, controls, and status indicators. |
| ACTIVE ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| OPERATION LOG | Displays real-time system data, controls, and status indicators. |
| NOx COMPLIANCE | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

EXH INLET [C]

UREA INJ [C]

SCR BYPASS [O]

SOOT BLW [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Selective Catalytic Reduction (SCR) — Technical Reference

### Regulatory Framework

#### MARPOL Annex VI — Prevention of Air Pollution from Ships

#### Regulation 13 — Nitrogen Oxides (NOx)

**Tier III** applies in NOx Tier III ECAs for engines with power output ≥130 kW

Tier III limit for n < 130 rpm: **3.4 g/kWh**

Applies to ships built on or after 1 January 2016 operating in:

- North American ECA (from 1 January 2016)

- US Caribbean Sea ECA (from 1 January 2016)

- Baltic Sea NOx ECA (from 1 January 2021)

- North Sea NOx ECA (from 1 January 2021)

#### NOx Technical Code 2008 (NTC 2008)

SCR system must be certified as part of the approved engine NOx configuration

EIAPP (Engine International Air Pollution Prevention) Certificate must be endorsed for SCR mode

On-board NOx Verification Procedure (ONVP) for Tier III confirmation

Urea consumption must be monitored and recorded

#### IACS UR M74 — NOx Monitoring

Continuous NOx monitoring required for Tier III systems

Urea injection rate cross-checked against engine load/NOx curve

Annual survey requirements for catalyst condition assessment

Ammonia slip detector mandatory: limit 10 ppm

---

### SCR Operating Principle

Selective Catalytic Reduction converts NOx to nitrogen (N₂) and water (H₂O) using:

**Reducing agent**: Urea solution (AdBlue/DEF — 32.5% aqueous urea)

**Catalyst**: Vanadium-titanium oxide (V₂O₅/TiO₂) or zeolite-based

**Reactions**:

- Urea thermolysis: CO(NH₂)₂ → NH₃ + HNCO

- Hydrolysis: HNCO + H₂O → NH₃ + CO₂

- Standard SCR: 4 NH₃ + 4 NO + O₂ → 4 N₂ + 6 H₂O

- Fast SCR: 2 NH₃ + NO + NO₂ → 2 N₂ + 3 H₂O

---

### Equipment Parameters

#### Catalyst Bed

| Parameter | Value |

|-----------|-------|

| Optimal temperature | 350–550°C |

| Minimum operating temp | 320°C |

| Maximum temp (catalyst damage) | 650°C |

| Catalyst type | V₂O₅/TiO₂ or Zeolite |

| NOx reduction efficiency | 85–95% |

| Life expectancy | 20,000–30,000 operating hours |

#### Urea Injection System

| Parameter | Value |

|-----------|-------|

| Urea solution concentration | 32.5% (AdBlue / ISO 22241) |

| Injection method | Compressed air-assisted nozzle |

| Dosing range | 0–50 kg/h |

| Urea tank capacity | 20–60 m³ (voyage dependent) |

| Low level alarm | 20% |

#### Ammonia Slip

| Parameter | Value |

|-----------|-------|

| Maximum allowed | <10 ppm (IACS UR M74) |

| Measurement | After catalyst, before stack |

| Cause of high slip | Over-dosing urea |

| Response | Reduce injection rate |

---

### Catalyst Preheating Sequence

The SCR catalyst must reach minimum operating temperature before urea injection begins:

1. Exhaust gas reaches SCR inlet

2. Catalyst heats from exhaust gas (may take 10–30 minutes depending on engine load)

3. Temperature sensor confirms >320°C

4. Catalyst preheating complete → SCR system READY

5. Begin urea injection at low rate

6. Gradually increase to target dose rate per NOx curve

Cold Start: If catalyst temperature is below 320°C:

**Bypass damper must be opened** — exhaust routed around catalyst

Avoid injecting urea at low temperature — causes catalyst fouling with ammonium bisulphate

Continue in Tier II mode (fuel-based compliance) until temperature achieved

---

### Performance Curve

Typical NOx reduction vs urea dose rate relationship:

| Dose Rate (kg/h) | NOx Reduction (%) | NOx Out (g/kWh) | NH₃ Slip (ppm) |

|-----------------|-----------------|-----------------|----------------|

| 0 | 0 | 14.0 | 0 |

| 5 | 35 | 9.1 | <1 |

| 10 | 60 | 5.6 | 2 |

| 20 | 78 | 3.1 | 4 |

| 30 | 86 | 2.0 | 7 |

| 40 | 88 | 1.7 | 12 (exceeds limit!) |

| 50 | 89 | 1.5 | 25 (far exceeds!) |

Target: NOx <3.4 g/kWh with NH₃ slip <10 ppm.

Optimal operating point: ~25–35 kg/h at typical engine load.

---

### Alarm Setpoints

| ID | Description | Setpoint | Type |

|----|-------------|----------|------|

| SCR-001 | High ammonia slip | >10 ppm | Alarm |

| SCR-002 | Low catalyst temperature — bypass | <320°C | Alarm |

| SCR-003 | High catalyst temperature | >620°C | Alarm |

| SCR-004 | NOx out of limits | >3.4 g/kWh (T3 ECA) | Alarm |

| SCR-005 | Urea tank low level | <20% | Warning |

| SCR-006 | Urea dosing pump fault | — | Alarm |

| SCR-007 | Soot blower fault | — | Warning |

| SCR-008 | NOx sensor fault | — | Warning |

---

### Operating Procedure

#### Tier III Activation (from Tier II)

1. Confirm ECA approach — minimum 1 hour before boundary.

2. Check catalyst temperature ≥320°C (normally achieved at >25% engine load).

3. Open SCR exhaust inlet valve.

4. Close SCR bypass valve.

5. Start urea dosing pump.

6. Open urea injection valve.

7. Gradually increase urea injection rate to target per engine NOx curve.

8. Confirm NOx after SCR <3.4 g/kWh within 10 minutes.

9. Monitor ammonia slip continuously — must remain <10 ppm.

10. Log Tier III activation in engine log and EDR.

#### Bypass Procedure (Low Temperature / Fault)

1. Close urea injection valve immediately.

2. Stop urea dosing pump.

3. Open SCR bypass valve.

4. Close SCR exhaust inlet valve.

5. Revert to Tier II operation (distillate or low-S HFO as applicable).

6. Log bypass event with reason and duration.

---

### Soot Blower Operation

Periodic cleaning of catalyst elements

Steam or compressed air soot blowing

Frequency: every 24–72 hours depending on soot load

During soot blowing: brief increase in particulate emission — log event

---

### References

MARPOL 73/78 Annex VI, Regulation 13 — NOx

NOx Technical Code 2008 (NTC 2008)

IACS UR M74 — NOx Monitoring Equipment

ISO 22241 — Diesel engine NOx reduction agents — AUS 32 (AdBlue)

Wärtsilä SCR System Technical Manual

MAN Energy Solutions SCR Engineering Guidelines

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System